Agility - Overhead Rings

Description Tasks Risk score 8

Innate risk score:

Timber is decayed. Check on a routine basis. 7

Additional comments are noted below. Read the notes for further action. 5

There are natural splits / shakes in the timbers.

Shakes vary. Monitor and consult manufacturer if dimensions exceed 8 mm at sections used to grip / grasp. 4

Action: Replace new unit and safety surface. YEAR 1 2026/27

Climber - Traverse Wall

Description Tasks Risk score 4

Innate risk score:

Timber is decayed. Check on a routine basis. 7

There are natural splits / shakes in the timbers.

Shakes vary. Monitor and consult manufacturer if dimensions exceed 8 mm at sections used to grip / grasp. 3

Action: Replace 3 x posts and safety surface. YEAR 1 2026/27

Climber - A-Frame

Description Tasks Risk score 5

Innate risk score:

Bolt cap damaged. Replace. 2

Item is cracked. Read the notes for further action. 2

Cap missing. Replace. 2

Action: Install metal feet and Replace safety surface  YEAR 1 2026/27

N.B THE SAFETY SURFACING FOR ALL 3 OF THE ABOVE IS CONTIGUOUS

Multiplay

Description Tasks Risk score 6

Innate risk score:

There are splinters or the potential for splinters.

Sand the timber down to remove the splinters and/or the places liable to give splinters. 4

Timber is decayed. Check on a routine basis. 3

Action: Consider replacing 4 2 x post and monitor shakes YEAR 2 2027/28

Climber - Frame & Net

Description Tasks Risk score 4

Innate risk score:

Surface needs repair. Repair. 4

Surface is uneven. Read the notes for further action. 3

Action: Replace safety surface YEAR 2 2027/28

Play Panel

Description Tasks Risk score 3

Innate risk score:

Surface is uneven. Read the notes for further action. 3

Fixtures loose or missing. Tighten. 1

Action: Replace unit plus safety surface – Year 2

Swing - Junior - 1 Bay 2 Seat

Description Tasks Risk score 5

Innate risk score:

There is wear in the bushes. Remove shackle bolt and check bush

and shackle pin wear, replacing as necessary. 6

Surface needs repair. Repair. 4

Loose swing seat eye-bolts. Tighten connecting nuts. 4

Cap missing. Replace. 3

Action: Routine Monitoring REPLACE SAFETY SURFACE YEAR 3 2028/29

Swing - Toddler - 1 Bay 2 Seat

Description Tasks Risk score 4

Innate risk score:

Cap missing. Replace. 5

Surface needs repair. Repair. 4

Action: Replace safety surface – YEAR 3 2028/29

Rocker - Horse

Description Tasks Risk score 4

Innate risk score:

Protruding handles / foot rests. The unit was installed prior to the

publication of the current standards.

There is no requirement to make retrospective changes. 3

Action: Replace plus safety surface  year 2/3   2027/28 OR 2028/29

Rocker - Bike

Description Tasks Risk score 4

Innate risk score:

Protruding handles / foot rests. The unit was installed prior to the

publication of the current standards.

There is no requirement to make retrospective changes. 3

Bolt cap damaged. Replace. 1

Action: Replace plus safety surface year 2/3  2027/28 OR 2028/29

Rocker - Seesaw

Description Tasks Risk score 4

Innate risk score:

Protruding handles / foot rests. The unit was installed prior to the

publication of the current standards.

There is no requirement to make retrospective changes. 3

Action: Replace plus safety surface year 2/3  2027/28 OR 2028/29